Sophos Firewall Monthly Activity

Brandywine Homes — January 2026

Reporting period: January 2026

Confidential — for internal Technijian and named-client use only.

|  | Executive Summary |
| --- | --- |

| 1 Firewalls | 1 Online | 93 Alerts (month) | 3 Alerts open | 0 Events |
| --- | --- | --- | --- | --- |

93 alert(s) were generated across the reporting period; 3 remain open. Review the Connectivity Alerts section for detail.

This report summarizes Sophos XGS firewall connectivity, security posture, and operational activity for Brandywine Homes during January 2026. Alert and event counts are aggregated from daily Sophos Central Partner API pulls; firewall configuration reflects the most recent on-box API snapshot (2026-05-01).

|  | Firewall Inventory |
| --- | --- |

| Hostname | Model | Firmware | Serial | WAN IP | Status |
| --- | --- | --- | --- | --- | --- |
| BWH-HQ-FW-01 | SFV2C4_VM01_SFOS 22.0.0 GA-Build411 | SF01V_VM01_22.0.0.411 | V010012FG8J8297 | 64.58.142.218, 98.174.153.35 | Online |

Firmware version distribution (across reporting month):

| Firmware major version | Days seen |
| --- | --- |
| 22.0 | 31 |

|  | Firewall Configuration |
| --- | --- |

Configuration snapshot captured 2026-05-01 via the Sophos XGS on-box API. Reflects the current state of interfaces, zones, routing, DHCP, NAT, and firewall rules.

Network Interfaces:

| Interface | IP Address | Netmask | Zone | Status |
| --- | --- | --- | --- | --- |
| PortA | 10.1.1.1 | 255.255.255.0 | - | Connected, 10000 Mbps - Full Duplex, FEC off |
| PortB | 64.58.142.218 | 255.255.255.248 | - | Connected, 10000 Mbps - Full Duplex, FEC off |
| PortC | 98.174.153.35 | 255.255.255.248 | - | Connected, 10000 Mbps - Full Duplex, FEC off |

Zones:

| Zone | Type | Description |
| --- | --- | --- |
| LAN | LAN | - |
| WAN | WAN | - |
| DMZ | DMZ | - |
| LOCAL | LOCAL | - |
| VPN | VPN | - |
| Discover | Discover | - |
| WiFi | LAN | - |

Firewall Rules (7 total):

| Rule Name | Action | Source Zone | Dest Zone | Enabled |
| --- | --- | --- | --- | --- |
| Bypass_Test | - | - | - | Yes |
| DNAT to BWH-HQ-IIS-01_1757265006699 | - | - | - | Yes |
| VPN to LAN | - | - | - | Yes |
| LAN to VPN | - | - | - | Yes |
| RingCentral | - | - | - | Yes |
| Auto added firewall policy for MTA | - | - | - | Yes |
| #Default_Network_Policy | - | - | - | Yes |

NAT Rules (8 total):

| Rule Name | Type | Original IP | Translated IP |
| --- | --- | --- | --- |
| DNAT to BWH-HQ-IIS-01_1757265006699 | - | - | - |
| Loopback_NAT#6_DNAT to BWH-HQ-IIS-01_1757265006699 | - | - | - |
| Reflexive_NAT#6_DNAT to BWH-HQ-IIS-01_175726500669 | - | - | - |
| VPN to LAN | - | - | - |
| LAN to VPN | - | - | - |
| #NAT_Default_Network_Policy | - | - | - |
| Auto added NAT rule for MTA | - | - | - |
| Default SNAT IPv4 | - | - | - |

DHCP Scopes:

| Scope Name | Interface | Start IP | End IP | Lease |
| --- | --- | --- | --- | --- |
| BWH-HQ | PortA | - | - | - |

Static Routes (1):

| Destination | Mask | Gateway | Interface | Metric |
| --- | --- | --- | --- | --- |
| - | - | - | - | - |

|  | Security Posture |
| --- | --- |

Current intrusion-prevention, web-filtering, SSL/TLS inspection, and network-object configuration from the most recent on-box snapshot. This section captures the security controls in place on the Sophos XGS firewall at time of last pull.

IPS / Intrusion Prevention Policies (10):

| Policy Name | Description |
| --- | --- |
| generalpolicy | A General Policy |
| lantowan_strict | A Strict policy for LAN to WAN Traffic |
| lantowan_general | A General policy for LAN to WAN Traffic |
| dmzpolicy | A General policy to scan traffic flowing to DMZ |
| WAN TO DMZ | A default IPS policy template to scan the traffic flowing from WAN to DMZ; Primarily intended to secure server(s) hosted in the DMZ |
| WAN TO LAN | A default IPS policy template to scan the traffic flowing from WAN to LAN; Primarily intended to secure server(s) hosted in the LAN |
| DMZ TO LAN | A default IPS policy template to scan the traffic flowing from DMZ to LAN; Primarily intended to secure server(s) hosted in the LAN zone |
| DMZ TO WAN | A default IPS policy template to scan the traffic flowing from DMZ to WAN; Primarily intended to secure the DMZ-based client(s) |
| LAN TO DMZ | A default IPS policy template to scan the traffic flowing from LAN to DMZ; Primarily intended to secure the LAN-based client(s) and DMZ-based server(s) |
| LAN TO WAN | A default IPS policy template to scan the traffic flowing from LAN to WAN; Primarily intended to secure LAN-based client(s) |

Web Filter Policies (12):

| Policy Name | Description |
| --- | --- |
| No Web Mail or Chat | Deny access to web mail and online chat sites |
| No Web Mail | Deny access to web mail sites |
| No Online Chat | Deny access to online chat sites |
| Allow All | Unrestricted web access |
| Deny All | All user web activity will be blocked |
| CIPA Compliance | Deny categories necessary for compliance with the Children's Internet Protection Act |
| No web uploads | Restrict users from uploading content to any site |
| Default Workplace Policy | Deny access to categories most commonly unwanted in professional environments |
| No Explicit Content | Deny access to sexually explicit sites |
| No Ads or Explicit Content | Deny access to advertisements and sexually explicit sites |
| No Games Ads or Explicit Content | Deny access to games, advertisements, and sexually explicit sites |
| Default Policy | A typical starter policy with options suitable for many organizations |

SSL/TLS Inspection Rules (1):

| Rule Name | Action | Status |
| --- | --- | --- |
| Exclusions by website or category | - | - |

| 34 IP Host Objects | 1 Host Groups | 62 Service Objects |
| --- | --- | --- |

|  | Connectivity Alerts |
| --- | --- |

Total alerts generated by Sophos Central during January 2026: 93. Sophos Central alerts include connectivity events (gateway up/down), license notifications, and policy violations.

| 93 Total alerts | 3 Open alerts | 31 Days with connectivity alerts |
| --- | --- | --- |

Alerts by category:

| Category | Count |
| --- | --- |
| Connectivity | 2 |
| Updating | 1 |

Alerts by severity:

| Severity | Count |
| --- | --- |
| High | 31 |
| Low | 62 |

Alerts by product:

| Product | Count |
| --- | --- |
| Firewall | 62 |
| Other | 31 |

Top alert descriptions:

| Alert Description | Count |
| --- | --- |
| Firewall has not contacted Sophos Central for the past 5 minutes | 31 |
| Firewall connection to Sophos Central is restored | 31 |
| A firmware update has completed successfully, and the firewall is now ready to ... | 31 |

|  | Firewall Events |
| --- | --- |

Sophos Central event log entries for the reporting period: 0 total. Events include policy matches, threat detections, and system activity recorded by the firewall and reported to Sophos Central.

No firewall events were recorded in Sophos Central during this period. This is typical for firewalls where only connectivity-level alerts are forwarded to Sophos Central. Detailed threat events require syslog forwarding to be configured to the Technijian DC syslog receiver.

|  | Daily Trend |
| --- | --- |

Day-by-day count of Sophos Central alerts and firewall events throughout the reporting month. Connectivity spikes typically indicate WAN instability or ISP outages. Persistent open alerts warrant investigation.

| Date | Alerts | Events |
| --- | --- | --- |
| 2026-01-01 | 3 | 0 |
| 2026-01-02 | 3 | 0 |
| 2026-01-03 | 3 | 0 |
| 2026-01-04 | 3 | 0 |
| 2026-01-05 | 3 | 0 |
| 2026-01-06 | 3 | 0 |
| 2026-01-07 | 3 | 0 |
| 2026-01-08 | 3 | 0 |
| 2026-01-09 | 3 | 0 |
| 2026-01-10 | 3 | 0 |
| 2026-01-11 | 3 | 0 |
| 2026-01-12 | 3 | 0 |
| 2026-01-13 | 3 | 0 |
| 2026-01-14 | 3 | 0 |
| 2026-01-15 | 3 | 0 |
| 2026-01-16 | 3 | 0 |
| 2026-01-17 | 3 | 0 |
| 2026-01-18 | 3 | 0 |
| 2026-01-19 | 3 | 0 |
| 2026-01-20 | 3 | 0 |
| 2026-01-21 | 3 | 0 |
| 2026-01-22 | 3 | 0 |
| 2026-01-23 | 3 | 0 |
| 2026-01-24 | 3 | 0 |
| 2026-01-25 | 3 | 0 |
| 2026-01-26 | 3 | 0 |
| 2026-01-27 | 3 | 0 |
| 2026-01-28 | 3 | 0 |
| 2026-01-29 | 3 | 0 |
| 2026-01-30 | 3 | 0 |
| 2026-01-31 | 3 | 0 |

|  | What Technijian Did For You |
| --- | --- |

Throughout the reporting period the Technijian security operations team performed the following ongoing services for your Sophos firewall environment:

24x7 monitoring: continuous health checks against every Sophos XGS firewall via the Sophos Central Partner API; alerts are reviewed within on-call SLA.

Connectivity tracking: daily firewall connection status is recorded; gaps beyond 15 minutes trigger an investigation and ticket in the Technijian Client Portal.

Configuration baseline: on-box API snapshots capture firewall rules, NAT, IPS policies, web filter, zones, interfaces, and network objects; deviations from the documented baseline are reviewed on each pull.

Firmware awareness: firmware version is tracked daily; Sophos XGS advisories are reviewed and upgrade windows are coordinated with you when critical patches are available.

Monthly reporting: this document is generated from the Sophos Central API and on-box configuration data, verified through a structural proofreader, and delivered as part of your Managed Security service.

Specific work delivered this month

These items are pulled directly from the time entries and tickets logged by Technijian's team during the period — actual work, not estimates.

Micheal williams from  BWH called in regarding the Sophos VPN issue (handled by S. Kumar) — 0.6h of tech effort

SOPHOS service unavailable (handled by S. Kumar) — 0.8h of tech effort

Sophos VPN not working - Nancy called (handled by G. Chamoli) — 0.7h of tech effort

SOPHOS service unavailable — 0.8h on 2026-01-07

Sophos VPN not working - Nancy called — 0.7h on 2026-01-07

RDP Access Issue on Nancy's System — 1.1h on 2026-01-07

|  | Industry News & Vendor Innovations |
| --- | --- |

Sophos announced no major platform changes this month. Routine detection-content updates and sensor improvements continued to roll out automatically. Technijian tracks Sophos's release notes monthly so future updates land in this section as they're published.

|  | Compliance Alignment |
| --- | --- |

This client's industry classification is Construction / Homebuilding. Based on that classification, the data Technijian's stack is helping protect is most likely characterized as: PII (employees, customers).

Construction firms typically have minimal regulatory compliance obligations beyond general business privacy law (CCPA for California-resident data, OSHA for safety records). Government contracts (DoD, DOE) bring CMMC or NIST 800-171 into scope when applicable.

How Technijian's monitoring stack supports each framework:

California CCPA: breach-detection and incident-response capabilities support CCPA §1798.150 reasonable-security obligations and the 72-hour notification window for incidents involving California-resident PII.

This classification was generated automatically from the client's name and contact-domain signals. If the industry or compliance scope shown here doesn't match your operations, please reply to this report and we'll update the classification — Technijian's team can also produce a deeper compliance-posture report on request.

|  | Recommendations |
| --- | --- |

Investigate connectivity gaps: the firewall lost contact with Sophos Central on 31 day(s) this period. Sustained outages may indicate WAN instability or ISP issues. Review ISP SLA and consider a secondary WAN link.

Resolve open alerts: 3 alert(s) remain open in Sophos Central. Log into the Central dashboard and acknowledge or dismiss any stale alerts to keep the alert queue clean.

Configure syslog forwarding: Sophos Central reports zero firewall events. IPS and threat events are only surfaced in Central if syslog forwarding is enabled. Configure Log Settings to forward to the Technijian DC syslog receiver for full threat-event visibility in future reports.

|  | About This Report |
| --- | --- |

This report is generated automatically from the Sophos Central Partner API and the Sophos XGS on-box configuration API by Technijian's annual-client-review pipeline. Alert and event counts are aggregated from daily API pulls; configuration data reflects the most recent on-box snapshot.

Report generated 2026-05-02 19:59 for client 'BWH'. Configuration snapshot: 2026-05-01.

For questions about this report or to request a different reporting cadence, email support@technijian.com.