Meraki Monthly Activity

Technijian Inc — March 2026

Reporting period: March 2026

Confidential — for internal Technijian and named-client use only.

|  | Executive Summary |
| --- | --- |

| 5 Networks | 12 Devices | 11,264 IDS/IPS events | 3,962 Activity events | 296 Config changes |
| --- | --- | --- | --- | --- |

Detect-mode activity — 11,264 alerts recorded with 0 blocks. Review whether the appliance should be moved from detect to prevention mode for high-priority signatures.

This report summarizes Cisco Meraki firewall, intrusion-prevention (IDS/IPS), advanced-malware-protection (AMP), and configuration posture for Technijian Inc during 2026-03. Activity counts are aggregated from daily Meraki Dashboard API pulls; configuration reflects the most recent snapshot.

|  | Network & Device Inventory |
| --- | --- |

Device inventory:

| Device | Network | Model | Serial | Firmware | LAN IP | Type |
| --- | --- | --- | --- | --- | --- | --- |
| TE-DC-FW-01 (Primary) | 01-Technijian DC | MX105 | Q3LB-2Y33-86PD | wired-19-2-4 | — | appliance |
| TE-DC-FW-01 (Secondary) | 01-Technijian DC | MX105 | Q3LB-HH3E-CJYD | wired-19-2-4 | — | appliance |
| TE-LAB-FW-01 | 05-Technijian LAB | MX105 | Q2WN-P4VJ-ZPR6 | wired-19-1-11 | — | appliance |
| TE-LAB-FW-01 (Spare) | 03-Technijian LV | MX100 | Q2JN-K5UD-GNPQ | wired-18-1-07 | — | appliance |
| TE-LV-FW-01 (Primary) | 03-Technijian LV | MX100 | Q2JN-6K9Q-M6K2 | wired-18-1-07 | — | appliance |
| LV-DC-Active | 03-Technijian LV | MS42 | Q2CP-7MQ9-GUYF | switch-15-21-1 | 192.168.254.2 | switch |
| LV-DC-FailOver | 03-Technijian LV | MS42 | Q2CP-7VLB-8TNC | switch-15-21-1 | 192.168.253.10 | switch |
| TE-HQ-WAREHOUSE-01 | 02-Technijian HQ | MS42P | Q2DP-XDND-URS4 | Not running configured version | 10.10.10.134 | switch |
| TE-HQ-AP-01 | 02-Technijian HQ | MR42 | Q2KD-CFPE-EXBF | wireless-30-7-1 | 10.120.1.15 | wireless |
| TE-HQ-AP-02 | 02-Technijian HQ | MR42 | Q2KD-CENM-CZUL | wireless-30-7-1 | 10.120.1.16 | wireless |
| TE-HQ-AP-03 | 02-Technijian HQ | MR66 | Q2CK-DWQ4-MUAR | wireless-26-8-3 | 10.120.1.17 | wireless |
| TE-HQ-AP-04 | 02-Technijian HQ | MR42 | Q2KD-CQUG-H59C | wireless-30-7-1 | 10.120.1.26 | wireless |

Networks under management:

| Network | Product types | VLANs | L3 rules | L7 rules | Inbound | Port fwd |
| --- | --- | --- | --- | --- | --- | --- |
| 01-Technijian DC | appliance | 7 | 7 | 1 | 0 | 0 |
| 02-Technijian HQ | sensor, switch, wireless | 0 | 0 | 0 | 0 | 0 |
| 03-Technijian LV | appliance, switch | 10 | 1 | 0 | 0 | 0 |
| 04-Technijian MDM | systemsManager | 0 | 0 | 0 | 0 | 0 |
| 05-Technijian LAB | appliance | 1 | 1 | 0 | 1 | 0 |

|  | Firewall Configuration |
| --- | --- |

Full configuration snapshot of WAN interfaces, firewall rules, VLANs, port-forwarding, wireless SSIDs, and site-to-site VPN per network. Values reflect the most recent configuration pull.

WAN Interface Configuration:

| Device | Network | Interface | Mode | IP / Subnet | Gateway | DNS |
| --- | --- | --- | --- | --- | --- | --- |
| TE-DC-FW-01 (Primary) | 01-Technijian DC | WAN1 | Static | 66.81.19.126/25 | 66.81.19.1 | 1.1.1.1 / 1.0.0.1 |
| TE-DC-FW-01 (Secondary) | 01-Technijian DC | WAN1 | Static | 66.81.18.194/28 | 66.81.18.193 | 1.1.1.1 / 4.2.2.4 |
| TE-LAB-FW-01 | 05-Technijian LAB | WAN1 | Static | 10.120.50.109/24 (failed) | 10.120.50.1 | 8.8.8.8 / 8.8.4.4 |
|  |  | WAN2 | Static | 99.50.93.145/29 (not connected) | 99.50.93.150 | 8.8.8.8 / 8.8.4.4 |
| TE-LAB-FW-01 (Spare) | 03-Technijian LV | WAN1 | Static | 10.120.50.253/24 (not connected) | 10.120.50.1 | 8.8.8.8 / 4.2.2.4 |
| TE-LV-FW-01 (Primary) | 03-Technijian LV | WAN1 | Static | 10.120.50.252/24 (not connected) | 10.120.50.1 | 4.2.2.4 / 8.8.8.8 |

01-Technijian DC

L3 Outbound Firewall Rules:

| Policy | Proto | Source | Destination | Comment |
| --- | --- | --- | --- | --- |
| Allow | TCP | 10.95.10.0/24 | Any | SSH to 3850 – mgmt IP |
| Deny | ANY | Any | cvc.7766.org | Blocked Malicious website outbound connection |
| Deny | ANY | Any | 209.141.33.145/32 | Blocked Malicious website IP outbound connection |
| Deny | ANY | Any | 209.141.45.118/32 | Blocked Malicious website IP outbound connection |
| Allow | ANY | Any | 3.218.175.179/32 | cavelo.com |
| Allow | TCP | Any | api.anthropic.com,claude.ai,api.openai.com,codex.openai.com,generativelanguage.googleapis.com,api.moonshot.ai,api.deepseek.com,api.z.ai,api.minimax.io,z.ai | Whitelist Domains |
| Allow | ANY | Any | Any | Default rule |

VLANs:

| ID | Name | Subnet | Gateway | DHCP |
| --- | --- | --- | --- | --- |
| 1 | VLAN-001-MNG | 10.100.1.0/24 | 10.100.1.1 | Disabled |
| 49 | VLAN-049-Transist | 10.100.49.0/24 | 10.100.49.254 | Disabled |
| 115 | VLAN-115-ISOLATION TEST | 10.100.115.0/24 | 10.100.115.1 | DHCP server |
| 116 | VLAN-116-ISOLATION TEST | 10.100.116.0/24 | 10.100.116.4 | DHCP server |
| 250 | VLAN-250-SEO-TEST | 10.100.250.0/24 | 10.100.250.1 | Disabled |
| 251 | VLAN-251-SEO-PROD | 10.100.251.0/24 | 10.100.251.1 | Disabled |
| 252 | VLAN-252-DEV-TEST | 10.100.252.0/24 | 10.100.252.1 | Disabled |

Site-to-Site VPN — Mode: Hub

| Local Subnet | In VPN |
| --- | --- |
| 10.100.10.0/24 | No |
| 10.100.1.0/24 | No |
| 10.100.49.0/24 | No |
| 10.100.115.0/24 | No |
| 10.100.116.0/24 | No |
| 10.100.250.0/24 | No |
| 10.100.251.0/24 | No |
| 10.100.252.0/24 | No |
| 172.16.5.252/30 | No |
| 10.100.240.0/24 | No |
| 10.100.103.0/24 | No |
| 10.100.104.0/24 | No |
| 10.100.105.0/24 | No |
| 10.100.106.0/24 | No |
| 10.100.109.0/24 | No |
| 10.100.100.0/24 | No |
| 10.100.110.0/24 | No |
| 66.81.19.108/32 | No |
| 10.100.112.0/24 | No |
| 10.100.113.0/24 | No |
| 10.100.114.0/24 | No |
| 172.16.9.0/24 | No |
| 10.100.254.0/24 | No |
| 10.100.5.0/24 | No |
| 10.100.12.0/24 | No |
| 10.100.13.0/24 | No |
| 10.100.14.0/24 | No |
| 10.7.9.0/24 | No |
| 10.100.101.0/24 | No |
| 10.100.253.0/24 | No |

Content Filtering:

Blocked URL Pattern

cvc.7766.org

02-Technijian HQ

Wireless SSIDs (enabled):

| # | SSID Name | Auth Mode |
| --- | --- | --- |
| — | TechnijianSecure | PSK |
| 2 | Technijian HQ | 8021x-radius |
| 3 | TechnijianGuest | PSK |

03-Technijian LV

L3 Outbound Firewall Rules:

| Policy | Proto | Source | Destination | Comment |
| --- | --- | --- | --- | --- |
| Allow | ANY | Any | Any | Default rule |

VLANs:

| ID | Name | Subnet | Gateway | DHCP |
| --- | --- | --- | --- | --- |
| 1 | VLAN-001-MNG | 10.110.1.0/24 | 10.110.1.1 | Disabled |
| 2 | VLAN-002-OTB | 10.110.2.0/24 | 10.110.2.1 | Disabled |
| 3 | VLAN-003-VOI | 10.110.3.0/24 | 10.110.3.1 | Disabled |
| 4 | VLAN-004-IOT | 10.110.4.0/24 | 10.110.4.1 | Disabled |
| 5 | VLAN-005-RSV | 10.110.5.0/24 | 10.110.5.1 | Disabled |
| 6 | VLAN-006-STR | 10.110.6.0/24 | 10.110.6.1 | Disabled |
| 7 | VLAN-007-BKP | 10.110.7.0/24 | 10.110.7.1 | Disabled |
| 8 | VLAN-008-VMT | 10.110.8.0/24 | 10.110.8.1 | Disabled |
| 9 | VLAN-009-VHA | 10.110.9.0/24 | 10.110.9.1 | Disabled |
| 254 | VLAN-254-TECHNIJIAN | 10.110.254.0/24 | 10.110.254.1 | DHCP server |

Site-to-Site VPN — Mode: Hub

| Local Subnet | In VPN |
| --- | --- |
| 10.110.1.0/24 | Yes |
| 10.110.2.0/24 | No |
| 10.110.3.0/24 | No |
| 10.110.4.0/24 | No |
| 10.110.5.0/24 | No |
| 10.110.6.0/24 | No |
| 10.110.7.0/24 | No |
| 10.110.8.0/24 | No |
| 10.110.9.0/24 | No |
| 10.110.254.0/24 | Yes |

04-Technijian MDM

05-Technijian LAB

L3 Outbound Firewall Rules:

| Policy | Proto | Source | Destination | Comment |
| --- | --- | --- | --- | --- |
| Allow | ANY | Any | Any | Default rule |

Inbound Firewall Rules:

| Policy | Proto | Source | Destination | Comment |
| --- | --- | --- | --- | --- |
| Deny | ANY | Any | Any | Default rule |

VLANs:

| ID | Name | Subnet | Gateway | DHCP |
| --- | --- | --- | --- | --- |
| 49 | VLAN - 49 | 10.100.49.0/24 | 10.100.49.254 | DHCP server |

|  | Security Posture |
| --- | --- |

Current configuration of the intrusion-prevention engine, anti-malware protection, content filtering, site-to-site VPN, and syslog forwarding per network.

| Network | IDS/IPS mode | AMP mode | URL cats blocked | URL patterns blocked | S2S VPN | Syslog dests |
| --- | --- | --- | --- | --- | --- | --- |
| 01-Technijian DC | Detection | Enabled | 0 | 1 | Hub | 0 |
| 02-Technijian HQ | — | — | 0 | 0 | — | 0 |
| 03-Technijian LV | Disabled | Disabled | 0 | 0 | Hub | 0 |
| 04-Technijian MDM | — | — | 0 | 0 | — | 0 |
| 05-Technijian LAB | Disabled | Disabled | 0 | 0 | None | 0 |

|  | Configuration Changes |
| --- | --- |

| 296 Total changes | 7 Admins active | 2 Networks affected |
| --- | --- | --- |

296 Dashboard configuration changes were recorded across 2 network(s) by 7 administrator(s). The tables below identify who changed what and provide the full before/after detail for compliance review.

Changes by administrator:

| Administrator | Changes |
| --- | --- |
| pbiswal@technijian.com | 223 |
| srevanth@Technijian.com | 30 |
| abhardwaj@technijian.com | 16 |
| skumar@technijian.com | 11 |
| gkumar@technijian.com | 9 |
| Asaraf@technijian.com | 4 |
| support@technijian.com | 3 |

Changes by network:

| Network | Changes |
| --- | --- |
| 05-Technijian LAB | 234 |
| 01-Technijian DC | 62 |

Changes by configuration area:

| Configuration area | Changes |
| --- | --- |
| Addressing & VLANs | 229 |
| Firewall | 67 |

Change detail (most recent 25):

| Date / Time | Administrator | Area: Setting | Before | After |
| --- | --- | --- | --- | --- |
| 2026-03-31 20:00 | support@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-31 02:01 | Asaraf@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-31 02:00 | Asaraf@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-30 00:33 | support@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-30 00:22 | support@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-29 23:50 | abhardwaj@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-29 23:50 | abhardwaj@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-26 23:37 | pbiswal@technijian.com | Firewall: 1:1 NAT | [] | [{"Name":"TE-HQ-LAB-IIS","wanip":"10.120.50.200","lanip":"10.100.254.21","uplin… |
| 2026-03-26 10:02 | Asaraf@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-26 10:01 | Asaraf@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Per Port VLAN setting peer SGT capablility enabled with port: |  | false |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Per Port VLAN setting native VLAN was changed with port: |  | VLAN 49 |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Per port VLAN setting type was changed with port: |  | true |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Per Port VLAN setting allowed VLANs was changed with port: |  | ["all"] |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Per port VLAN settings enabled with port: | false | true |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: VLAN was created |  |  |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Staticroutes Guid |  | 64 |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Static route enabled |  | true |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Static route conditions was changed |  | always |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Static route gateway IP was changed |  | 10.100.49.1 |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Static route subnet was changed |  | 10.100.1.0/24 |
| 2026-03-25 20:34 | pbiswal@technijian.com | Addressing & VLANs: Static route name was changed |  | VLAN - Management |
| 2026-03-25 20:33 | pbiswal@technijian.com | Addressing & VLANs: VLAN was deleted with ID: |  |  |
| 2026-03-25 20:33 | pbiswal@technijian.com | Addressing & VLANs: Per port VLAN settings enabled with port: | true | false |
| 2026-03-25 09:52 | abhardwaj@technijian.com | Firewall: 1:1 NAT | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… | [{"Name":"alpha-mysms.technijian.com","lanip":"10.100.254.71","uplink":"1","all… |

|  | IDS/IPS & AMP Events |
| --- | --- |

Total IDS/IPS and AMP events captured during the reporting period: 11,264 across 6 days with activity.

Blocked vs. alerted:

| Action | Events |
| --- | --- |
| Alerted | 11,264 |

By priority / severity:

| Priority | Events |
| --- | --- |
| 1 | 11,188 |
| 2 | 43 |
| 3 | 33 |

Top signatures:

| Signature | Hits |
| --- | --- |
| 1:45749:2 | 8,889 |
| 1:300053:3 | 733 |
| 1:63598:1 | 695 |
| 1:65554:1 | 476 |
| 1:65183:1 | 98 |
| 1:58853:1 | 84 |
| 1:300786:1 | 64 |
| 1:25977:4 | 47 |
| 1:51390:2 | 43 |
| 1:37732:6 | 33 |
| 3:64719:1 | 32 |
| 1:58992:1 | 22 |
| 1:25975:3 | 20 |
| 1:44687:3 | 15 |
| 1:300677:1 | 2 |

Top source IPs:

| Source | Hits |
| --- | --- |
| 149.112.84.49:12345 | 33 |
| 62.84.179.244:55314 | 32 |
| 147.93.146.163:38332 | 32 |
| 185.213.27.243:52848 | 32 |
| 75.119.143.158:55738 | 32 |
| 61.245.11.87:43780 | 32 |
| 103.40.61.98:54036 | 32 |
| 103.40.61.98:46984 | 32 |
| 87.106.91.226:60836 | 32 |
| 13.83.90.155:40734 | 32 |
| 77.237.237.57:35806 | 32 |
| 84.247.190.4:51286 | 32 |
| 185.91.69.217:41726 | 32 |
| 199.195.248.31:50610 | 32 |
| 45.88.223.138:59572 | 32 |

Top destination IPs:

| Destination | Hits |
| --- | --- |
| 10.100.253.51:80 | 904 |
| 10.100.254.71:80 | 780 |
| 10.100.251.99:80 | 616 |
| 10.100.103.53:80 | 561 |
| 10.100.106.65:80 | 524 |
| 10.100.106.75:80 | 519 |
| 10.100.250.125:80 | 490 |
| 10.100.240.65:80 | 486 |
| 10.100.240.161:80 | 439 |
| 10.100.253.53:80 | 436 |
| 10.100.254.96:80 | 404 |
| 10.100.254.229:80 | 400 |
| 10.100.240.156:80 | 383 |
| 10.100.254.107:80 | 370 |
| 10.100.251.41:80 | 347 |

|  | Firewall & Network Activity |
| --- | --- |

Total firewall, VPN, DHCP, and connectivity events: 3,962 across 1 active networks.

Top event types:

| Event type | Count |
| --- | --- |
| martian_vlan | 2,078 |
| ip_conflict | 1,552 |
| route_connection_change | 106 |
| carrier_event | 92 |
| dhcp_lease | 80 |
| events_dropped2 | 30 |
| dhcp_release | 19 |
| dhcp_problem | 5 |

By category:

| Category | Count |
| --- | --- |
| ip_conflicts | 3,630 |
| route_tracking | 106 |
| dhcp | 104 |
| node_state | 92 |
| dropped | 30 |

Per-network rollup:

| Network | Events | Top event types |
| --- | --- | --- |
| 05-Technijian LAB | 3,962 | martian_vlan (2,078), ip_conflict (1,552), route_connection_change (106) |

|  | Daily Trend |
| --- | --- |

Day-by-day count of intrusion-prevention events versus appliance-layer activity events. Use this to spot anomalies — sudden spikes typically correlate with outbound malware activity, port scans, or VPN tunnel flapping.

| Date | Security events | Activity events |
| --- | --- | --- |
| 2026-03-01 | 0 | 349 |
| 2026-03-02 | 0 | 348 |
| 2026-03-03 | 0 | 265 |
| 2026-03-04 | 0 | 220 |
| 2026-03-05 | 0 | 200 |
| 2026-03-06 | 0 | 213 |
| 2026-03-07 | 0 | 194 |
| 2026-03-08 | 0 | 352 |
| 2026-03-09 | 0 | 334 |
| 2026-03-10 | 0 | 288 |
| 2026-03-11 | 0 | 248 |
| 2026-03-12 | 0 | 320 |
| 2026-03-13 | 0 | 184 |
| 2026-03-14 | 0 | 0 |
| 2026-03-15 | 0 | 0 |
| 2026-03-16 | 0 | 0 |
| 2026-03-17 | 0 | 19 |
| 2026-03-18 | 0 | 29 |
| 2026-03-19 | 0 | 112 |
| 2026-03-20 | 0 | 108 |
| 2026-03-21 | 0 | 30 |
| 2026-03-22 | 0 | 0 |
| 2026-03-23 | 0 | 0 |
| 2026-03-24 | 0 | 12 |
| 2026-03-25 | 0 | 136 |
| 2026-03-26 | 1,859 | 0 |
| 2026-03-27 | 1,877 | 0 |
| 2026-03-28 | 1,986 | 0 |
| 2026-03-29 | 2,310 | 0 |
| 2026-03-30 | 1,591 | 0 |
| 2026-03-31 | 1,641 | 1 |

|  | What Technijian Did For You |
| --- | --- |

Throughout the reporting period the Technijian network operations team performed the following ongoing services for your Meraki environment:

24x7 monitoring: continuous health checks against every Meraki appliance, switch, and access point in your dashboard.

Threat intelligence: Cisco's Talos and AMP feeds are kept current; signature definitions update automatically as Cisco publishes them.

Configuration drift detection: daily snapshots of firewall rules, VLANs, content filtering, IDS/IPS settings, and VPN configuration; deviations from the documented baseline are reviewed.

Incident review: high-priority security alerts are triaged within the on-call SLA and ticketed in the Technijian Client Portal.

Monthly reporting: this document — every line item is sourced from the Meraki Dashboard API and verified through a structural proofreader before delivery.

|  | Industry News & Vendor Innovations |
| --- | --- |

Cisco Meraki announced no major platform changes this month. Routine detection-content updates and sensor improvements continued to roll out automatically. Technijian tracks Cisco Meraki's release notes monthly so future updates land in this section as they're published.

|  | Recommendations |
| --- | --- |

Move IDS to prevention mode: the following network(s) currently run IDS/IPS in detection-only mode: 01-Technijian DC, 03-Technijian LV, 05-Technijian LAB. We recommend a two-week prevention-mode trial after reviewing this period's signature distribution to confirm no false-positive risk.

Enable AMP: advanced-malware-protection is not fully enabled on 03-Technijian LV, 05-Technijian LAB. AMP adds file-reputation lookups and retrospective detection on top of signature-based IDS — recommended for any network handling email or web traffic.

Forward syslog: appliances on 01-Technijian DC, 03-Technijian LV, 05-Technijian LAB are not configured to forward syslog. Pointing these at the Technijian central syslog receiver lets us perform per-signature historical analysis beyond the Meraki API's 31-day window.

Review configuration changes: 296 Dashboard configuration change(s) were recorded this period. See the Configuration Changes section for full before/after detail. Verify each change was authorized and documented in your change-management log.

|  | About This Report |
| --- | --- |

This report is generated automatically from the Cisco Meraki Dashboard API by Technijian's annual-client-review pipeline. Every figure shown is sourced from the API at report-generation time; no manual data entry is involved.

Report generated 2026-05-02 20:00 for client code 'technijian'. Configuration snapshot taken 2026-04-30T16:54:18Z.

For questions about any item in this report, or to request a different reporting cadence, email support@technijian.com.