Meraki Monthly Activity

Aranda Tooling — January 2026

Reporting period: January 2026

Confidential — for internal Technijian and named-client use only.

|  | Executive Summary |
| --- | --- |

| 1 Networks | 4 Devices | 0 IDS/IPS events | 0 Activity events | 92 Config changes |
| --- | --- | --- | --- | --- |

All clear — no IDS/IPS or AMP events were recorded across the reporting period. Continue normal monitoring.

This report summarizes Cisco Meraki firewall, intrusion-prevention (IDS/IPS), advanced-malware-protection (AMP), and configuration posture for Aranda Tooling during 2026-01. Activity counts are aggregated from daily Meraki Dashboard API pulls; configuration reflects the most recent snapshot.

|  | Network & Device Inventory |
| --- | --- |

Device inventory:

| Device | Network | Model | Serial | Firmware | LAN IP | Type |
| --- | --- | --- | --- | --- | --- | --- |
| — | Aranda Tooling | MX85 | Q2YN-DL9C-NJLH | wired-19-1-11 | — | appliance |
| IDF 1 | Aranda Tooling | MS130-24P | Q4CF-NZXH-RBF9 | switch-17-2-2 | 10.28.0.20 | switch |
| IDF 2 | Aranda Tooling | MS130-24P | Q4CF-3WSW-SPDC | switch-17-2-2 | 10.28.0.30 | switch |
| IDF 3 | Aranda Tooling | MS130-24P | Q4CF-QSCK-MCQD | switch-17-2-2 | 10.28.0.40 | switch |

Networks under management:

| Network | Product types | VLANs | L3 rules | L7 rules | Inbound | Port fwd |
| --- | --- | --- | --- | --- | --- | --- |
| Aranda Tooling | appliance, switch | 14 | 2 | 0 | 1 | 0 |

|  | Firewall Configuration |
| --- | --- |

Full configuration snapshot of WAN interfaces, firewall rules, VLANs, port-forwarding, wireless SSIDs, and site-to-site VPN per network. Values reflect the most recent configuration pull.

WAN Interface Configuration:

| Device | Network | Interface | Mode | IP / Subnet | Gateway | DNS |
| --- | --- | --- | --- | --- | --- | --- |
| — | Aranda Tooling | WAN1 | Static | 76.80.219.186/29 | 76.80.219.185 | 8.8.8.8 / 8.8.4.4 |
|  |  | WAN2 | Static | 47.179.43.50/29 | 47.179.43.49 | 74.40.74.40 / 8.8.8.8 |

Aranda Tooling

L3 Outbound Firewall Rules:

| Policy | Proto | Source | Destination | Comment |
| --- | --- | --- | --- | --- |
| Allow | TCP | 10.10.7.50/32:587 | smtp.office365.com:587 | SMTP |
| Allow | ANY | Any | Any | Default rule |

Inbound Firewall Rules:

| Policy | Proto | Source | Destination | Comment |
| --- | --- | --- | --- | --- |
| Deny | ANY | Any | Any | Default rule |

VLANs:

| ID | Name | Subnet | Gateway | DHCP |
| --- | --- | --- | --- | --- |
| 1 | Untagged | 10.10.0.0/16 | 10.10.0.1 | DHCP server |
| 2 | Config VLAN | 192.168.254.0/24 | 192.168.254.1 | DHCP server |
| 3 | Config VLAN2 | 192.168.0.0/24 | 192.168.0.1 | DHCP server |
| 10 | Office Management VLAN | 172.28.0.0/23 | 172.28.0.1 | Disabled |
| 20 | Office Wired Network | 172.28.2.0/23 | 172.28.2.1 | DHCP server |
| 30 | Office Wi-Fi Network | 172.28.4.0/23 | 172.28.4.1 | DHCP server |
| 40 | Office Wi-Fi Guest | 172.28.6.0/23 | 172.28.6.1 | DHCP server |
| 50 | Office Services | 172.28.8.0/23 | 172.28.8.1 | Disabled |
| 60 | Office IoT | 172.28.10.0/23 | 172.28.10.1 | DHCP server |
| 70 | Office Printers VLAN | 172.28.12.0/23 | 172.28.12.1 | DHCP server |
| 100 | OT Management VLAN | 10.28.0.0/23 | 10.28.0.1 | Disabled |
| 110 | OT Wired Network | 10.28.2.0/23 | 10.28.2.1 | DHCP server |
| 120 | OT Printers | 10.28.4.0/23 | 10.28.4.1 | Disabled |
| 200 | BGP | 198.19.0.64/30 | 198.19.0.66 | Disabled |

Site-to-Site VPN — Mode: Hub

| Local Subnet | In VPN |
| --- | --- |
| 17.28.20.0/24 | Yes |
| 10.10.0.0/16 | No |
| 192.168.254.0/24 | No |
| 192.168.0.0/24 | No |
| 172.28.0.0/23 | No |
| 172.28.2.0/23 | Yes |
| 172.28.4.0/23 | Yes |
| 172.28.6.0/23 | No |
| 172.28.8.0/23 | Yes |
| 172.28.10.0/23 | No |
| 172.28.12.0/23 | Yes |
| 10.28.0.0/23 | No |
| 10.28.2.0/23 | No |
| 10.28.4.0/23 | No |
| 198.19.0.64/30 | No |

|  | Security Posture |
| --- | --- |

Current configuration of the intrusion-prevention engine, anti-malware protection, content filtering, site-to-site VPN, and syslog forwarding per network.

| Network | IDS/IPS mode | AMP mode | URL cats blocked | URL patterns blocked | S2S VPN | Syslog dests |
| --- | --- | --- | --- | --- | --- | --- |
| Aranda Tooling | — | — | 0 | 0 | Hub | 0 |

|  | Configuration Changes |
| --- | --- |

| 92 Total changes | 3 Admins active | 2 Networks affected |
| --- | --- | --- |

92 Dashboard configuration changes were recorded across 2 network(s) by 3 administrator(s). The tables below identify who changed what and provide the full before/after detail for compliance review.

Changes by administrator:

| Administrator | Changes |
| --- | --- |
| robbyb@us.dsbj.com | 87 |
| wchin@mflex.com | 4 |
| Meraki Admin | 1 |

Changes by network:

| Network | Changes |
| --- | --- |
| Aranda Tooling | 91 |
| (org-wide) | 1 |

Changes by configuration area:

| Configuration area | Changes |
| --- | --- |
| Addressing & VLANs | 53 |
| Switch ports | 28 |
| Client VPN settings | 5 |
| VPN settings | 2 |
| via API | 2 |
| Administrators | 1 |
| Packet Capture | 1 |

Change detail (most recent 25):

| Date / Time | Administrator | Area: Setting | Before | After |
| --- | --- | --- | --- | --- |
| 2026-01-31 00:05 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 20 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-30 23:56 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 20 | {"Native VLAN":100} | {"Native VLAN":1} |
| 2026-01-30 21:34 | robbyb@us.dsbj.com | VPN settings: VPN subnets | [{"name":"Untagged","vlan_id":"1","subnet":"10.10.0.0/16","subnets":[{"ip_versi… | [{"name":"Untagged","vlan_id":"1","subnet":"10.10.0.0/16","subnets":[{"ip_versi… |
| 2026-01-30 21:32 | robbyb@us.dsbj.com | VPN settings: VPN subnets | [{"name":"Untagged","vlan_id":"1","subnet":"10.10.0.0/16","subnets":[{"ip_versi… | [{"name":"Untagged","vlan_id":"1","subnet":"10.10.0.0/16","subnets":[{"ip_versi… |
| 2026-01-30 19:34 | robbyb@us.dsbj.com | Switch ports: IDF 1 / 2 | {"Native VLAN":100} | {"Native VLAN":1} |
| 2026-01-30 18:24 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 20 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-30 18:24 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 23 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-30 18:23 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 23 | {"Native VLAN":100} | {"Native VLAN":1} |
| 2026-01-30 18:22 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 20 | {"Native VLAN":100} | {"Native VLAN":1} |
| 2026-01-30 18:16 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 23 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-30 18:15 | robbyb@us.dsbj.com | Switch ports: IDF 2 / 20 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-30 18:15 | robbyb@us.dsbj.com | Switch ports: IDF 1 / 2 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-30 15:13 | robbyb@us.dsbj.com | Switch ports: IDF 3 / 22 | {"Native VLAN":1} | {"Native VLAN":100} |
| 2026-01-29 17:33 | wchin@mflex.com | Client VPN settings: User's list | [] | [{"User description":"William","User email":"wmch@mflex.com","Account type":"gu… |
| 2026-01-29 17:33 | wchin@mflex.com | Client VPN settings: User's list | {"User description":"William Chin","User email":"wchin@mflex.com","Account type… | {"User description":"William Chin","User email":"wchin@mflex.com","Account type… |
| 2026-01-29 17:31 | wchin@mflex.com | Client VPN settings: User's list | [] | [] |
| 2026-01-29 17:31 | wchin@mflex.com | Client VPN settings: User's list | {"User description":"William Chin","User email":"wchin@mflex.com","Account type… | {"User description":"William Chin","User email":"wchin@mflex.com","Account type… |
| 2026-01-29 17:24 | robbyb@us.dsbj.com | Addressing & VLANs: Per Port VLAN setting peer SGT capablility enabled with port: |  | false |
| 2026-01-29 17:24 | robbyb@us.dsbj.com | Addressing & VLANs: Per Port VLAN setting allowed VLANs was changed with port: | ["all"] | ["686235993221225951"] |
| 2026-01-29 17:13 | robbyb@us.dsbj.com | Switch ports: IDF 3 / 16 | {"VLAN":20} | {"VLAN":1} |
| 2026-01-29 07:24 | robbyb@us.dsbj.com | Switch ports: IDF 3 / 14 | {"Type":"trunk","Native VLAN":1,"Allowed VLANs":"1-4094"} | {"Type":"access","VLAN":20} |
| 2026-01-29 07:23 | robbyb@us.dsbj.com | Switch ports: IDF 1 / 20 | {"Type":"trunk","Native VLAN":20} | {"Type":"access","VLAN":1} |
| 2026-01-29 07:23 | robbyb@us.dsbj.com | Switch ports: IDF 3 / 16 | {"Type":"trunk","Native VLAN":1} | {"Type":"access","VLAN":20} |
| 2026-01-29 07:22 | robbyb@us.dsbj.com | Switch ports: IDF 1 / 20 | {"Native VLAN":1} | {"Native VLAN":20} |
| 2026-01-29 04:26 | robbyb@us.dsbj.com | Addressing & VLANs: Per Port VLAN setting peer SGT capablility enabled with port: |  | false |

|  | IDS/IPS & AMP Events |
| --- | --- |

Total IDS/IPS and AMP events captured during the reporting period: 0 across 0 days with activity.

No security events were recorded during this period. This indicates either a clean network or that the IDS/IPS engine is in detection-only mode with no triggered signatures. Review the Security Posture section to confirm the intended configuration.

|  | Firewall & Network Activity |
| --- | --- |

Total firewall, VPN, DHCP, and connectivity events: 0 across 0 active networks.

No appliance-layer activity events were recorded for this period. Wireless-only networks (no MX appliance) and dormant sites typically show zero events here — verify the network topology in the inventory above if this is unexpected.

|  | Daily Trend |
| --- | --- |

Day-by-day count of intrusion-prevention events versus appliance-layer activity events. Use this to spot anomalies — sudden spikes typically correlate with outbound malware activity, port scans, or VPN tunnel flapping.

| Date | Security events | Activity events |
| --- | --- | --- |
| 2026-01-01 | 0 | 0 |
| 2026-01-02 | 0 | 0 |
| 2026-01-03 | 0 | 0 |
| 2026-01-04 | 0 | 0 |
| 2026-01-05 | 0 | 0 |
| 2026-01-06 | 0 | 0 |
| 2026-01-07 | 0 | 0 |
| 2026-01-08 | 0 | 0 |
| 2026-01-09 | 0 | 0 |
| 2026-01-10 | 0 | 0 |
| 2026-01-11 | 0 | 0 |
| 2026-01-12 | 0 | 0 |
| 2026-01-13 | 0 | 0 |
| 2026-01-14 | 0 | 0 |
| 2026-01-15 | 0 | 0 |
| 2026-01-16 | 0 | 0 |
| 2026-01-17 | 0 | 0 |
| 2026-01-18 | 0 | 0 |
| 2026-01-19 | 0 | 0 |
| 2026-01-20 | 0 | 0 |
| 2026-01-21 | 0 | 0 |
| 2026-01-22 | 0 | 0 |
| 2026-01-23 | 0 | 0 |
| 2026-01-24 | 0 | 0 |
| 2026-01-25 | 0 | 0 |
| 2026-01-26 | 0 | 0 |
| 2026-01-27 | 0 | 0 |
| 2026-01-28 | 0 | 0 |
| 2026-01-29 | 0 | 0 |
| 2026-01-30 | 0 | 0 |
| 2026-01-31 | 0 | 0 |

|  | What Technijian Did For You |
| --- | --- |

Throughout the reporting period the Technijian network operations team performed the following ongoing services for your Meraki environment:

24x7 monitoring: continuous health checks against every Meraki appliance, switch, and access point in your dashboard.

Threat intelligence: Cisco's Talos and AMP feeds are kept current; signature definitions update automatically as Cisco publishes them.

Configuration drift detection: daily snapshots of firewall rules, VLANs, content filtering, IDS/IPS settings, and VPN configuration; deviations from the documented baseline are reviewed.

Incident review: high-priority security alerts are triaged within the on-call SLA and ticketed in the Technijian Client Portal.

Monthly reporting: this document — every line item is sourced from the Meraki Dashboard API and verified through a structural proofreader before delivery.

|  | Recommendations |
| --- | --- |

Forward syslog: appliances on Aranda Tooling are not configured to forward syslog. Pointing these at the Technijian central syslog receiver lets us perform per-signature historical analysis beyond the Meraki API's 31-day window.

Validate detection coverage: zero security events for the period is unusual unless the network is genuinely low-traffic. Consider running a controlled test (e.g., EICAR file download from an isolated workstation) to confirm the IDS/IPS pipeline end-to-end.

Review configuration changes: 92 Dashboard configuration change(s) were recorded this period. See the Configuration Changes section for full before/after detail. Verify each change was authorized and documented in your change-management log.

|  | About This Report |
| --- | --- |

This report is generated automatically from the Cisco Meraki Dashboard API by Technijian's annual-client-review pipeline. Every figure shown is sourced from the API at report-generation time; no manual data entry is involved.

Report generated 2026-05-02 18:14 for client code 'arnd'. Configuration snapshot taken 2026-04-30T16:48:40Z.

For questions about any item in this report, or to request a different reporting cadence, email support@technijian.com.